BIOSAFETY CABINET

OPERATION CHECKLIST

North Dakota Department of Health

Division of Laboratory Services – Microbiology

2635 East Main Ave.

Bismarck, ND  58501-5520

701-328-6272

After Hours Case Manager:  1-800-477-2121

1.              BEFORE YOU BEGIN:

- Plan uninterrupted time

- Make sure UV light is off

- Set sash at proper height

- Make sure grill is unobstructed

- Turn fluorescent light and blower on; let fan run for at least 10 minutes

- Check air flow gauge and/or paper strips

1

2.    AFTER BSC HAS RUN 10 MINUTES:

- Appropriate PPE; lab coat, gloves, eye protection, respirator (as required by lab policy)

- Wipe interior surfaces with 10% bleach or other disinfectant as appropriate

- After 10 minutes (if bleach was used) wipe with 70% alcohol or water to avoid pitting

- Load required materials – Do not overcrowd

- No open flame

- Do not obstruct any air grills

Leave space between objects

4.               WHEN YOU’RE DONE:

- Dispose of waste in biohazard container

- Decontaminate reusable items with 10% bleach and remove from BSC

- Cover and decontaminate all trays, containers, etc., before removal

- Wipe interior surfaces with 10% bleach, followed by alcohol/water after 10 minutes

- UV light on for 10 minutes - optional

- Let BSC blower run 15 minutes after use

3.                 CONDUCTING WORK:

- Allow cabinet to purge with equipment in place for 10 minutes prior to beginning work

- All materials are behind sash and at least 4 inches behind front air grill

- Avoid rapid movements in and out of BSC

- Work from clean to dirty

- Use proper microbiologic, aseptic technique

- Decontaminate all spills & splatters with blower on

3

Insert your organization’s information here:

Facility Name

Facility Address

Laboratory Director

Safety/Biosafety Officer

Emergency Phone number, Etc.

*Customize template to accommodate each laboratory needs